| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 1 What does PFAS stand for? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 2 Why are PFAS called "Forever Chemicals"? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 3 What does PFOA stand for? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 4 What does PFOS stand for? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 5 What are common PFAS sources? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 6 PFAS concentrations are commonly reported in what unit? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 7 1 ppt equals what concentration in water? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 8 What laboratory methods are commonly used for PFAS? |

| BACK ¦ Card 2 They are highly persistent and resist natural breakdown. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 1 Per- and Polyfluoroalkyl Substances. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| --- | --- |
| BACK ¦ Card 4 Perfluorooctane Sulfonic Acid. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 3 Perfluorooctanoic Acid. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 6 ppt (parts per trillion) or ng/L. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 5 Firefighting foam (AFFF), industrial manufacturing, stain-resistant products, nonstick coatings, grease-resistant packaging. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 8 EPA Method 533 and EPA Method 537.1 using LC-MS/MS. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 7 Approximately 1 ng/L. Source: Deck: T5 Flash Cards - PFAS and MCLs |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 9 Name three common PFAS treatment technologies. | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 10 Does chlorination effectively remove PFAS? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 11 Current EPA MCL for PFOA? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 12 Current EPA MCL for PFOS? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 13 What PFAS are included in EPA's Hazard Index approach? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 14 What does MCL stand for? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 15 What does MCLG stand for? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 16 Is an MCL enforceable? |

| BACK ¦ Card 10 No. Standard chlorination is not considered a PFAS removal technology. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 9 Granular Activated Carbon (GAC), Ion Exchange (IX), Reverse Osmosis/Nanofiltration. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| --- | --- |
| BACK ¦ Card 12 4 ppt / 4 ng/L under EPA's 2024 final PFAS NPDWR. Recheck before exam day because EPA announced proposed PFAS rule changes in 2026. Source: Water Operator Source Bibliography. | BACK ¦ Card 11 4 ppt / 4 ng/L under EPA's 2024 final PFAS NPDWR. Recheck before exam day because EPA announced proposed PFAS rule changes in 2026. Source: Water Operator Source Bibliography. |
| BACK ¦ Card 14 Maximum Contaminant Level. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 13 Under the 2024 final PFAS NPDWR: PFHxS, PFNA, HFPO-DA (GenX), and PFBS. This is date-sensitive because EPA announced proposed reconsideration/rescission of some non-PFOA/PFOS provisions in 2026. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 16 Yes. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 15 Maximum Contaminant Level Goal. Source: Deck: T5 Flash Cards - PFAS and MCLs |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 17 Is an MCLG enforceable? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 18 What federal law authorizes drinking water MCLs? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 19 Who establishes national drinking water standards? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 20 Who enforces California drinking water regulations? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 21 Current nitrate MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 22 Current nitrite MCL? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 23 Combined nitrate + nitrite rule? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 24 Arsenic MCL? |

| BACK ¦ Card 18 Safe Drinking Water Act (SDWA). Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 17 No. It is a health-based goal. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| --- | --- |
| BACK ¦ Card 20 California State Water Resources Control Board Division of Drinking Water. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 19 EPA. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 22 1 mg/L as nitrogen. Source: Water Operator Source Bibliography. | BACK ¦ Card 21 10 mg/L as nitrogen. Source: Water Operator Source Bibliography. |
| BACK ¦ Card 24 10 µg/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 23 Combined ratio cannot exceed 1.0. Verify the exact current regulatory wording during final regulation audit. Source: Deck: T5 Flash Cards - PFAS and MCLs |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 25 Fluoride MCL (federal)? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 26 Lead action level? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 27 Copper action level? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 28 Total Coliform Rule primarily addresses what concern? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 29 What is the MCL for E. coli? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 30 Gross Alpha MCL? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 31 Combined Radium 226/228 MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 32 Uranium MCL? |

| BACK ¦ Card 26 Lead uses an Action Level, not a standard MCL. Older/current LCR summary material commonly lists 15 ppb / 15 µg/L, while EPA's newer NPDWR/LCRI context lists 0.010 mg/L / 10 µg/L. Verify the current California exam expectation and implementation date before exam day. See Lead and Copper Rule Transition Note. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 25 4.0 mg/L. Source: Water Operator Source Bibliography. |
| --- | --- |
| BACK ¦ Card 28 Microbial contamination indicators. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 27 1.3 mg/L. Source: Water Operator Source Bibliography and Water Operator Source Bibliography. |
| BACK ¦ Card 30 15 pCi/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 29 Presence is generally unacceptable and triggers acute compliance actions. Verify exact rule wording during RTCR audit. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 32 30 µg/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 31 5 pCi/L. Source: Water Operator Source Bibliography. |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 33 Total Trihalomethanes (TTHM) MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 34 Haloacetic Acids (HAA5) MCL? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 35 Bromate MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 36 What does GAC stand for? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 37 How does GAC remove PFAS? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 38 Primary concern with GAC systems? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 39 Primary concern with ion exchange systems? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 40 Primary concern with RO systems? |

| BACK ¦ Card 34 60 µg/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 33 80 µg/L. Source: Water Operator Source Bibliography. |
| --- | --- |
| BACK ¦ Card 36 Granular Activated Carbon. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 35 10 µg/L. Source: Water Operator Source Bibliography. |
| BACK ¦ Card 38 Media breakthrough. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 37 Adsorption. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 40 Concentrate disposal and membrane maintenance. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 39 Resin exhaustion. Source: Deck: T5 Flash Cards - PFAS and MCLs |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 41 What is breakthrough? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 42 What is blending? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 43 Does blending remove contaminant mass? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 44 What treatment technology generally provides the highest PFAS removal? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 45 Which PFAS are often more difficult for adsorptive media to remove? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 46 What are microplastics? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 47 Are microplastics real and measurable? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 48 Do microplastics currently have a federal drinking water MCL? |

| BACK ¦ Card 42 Mixing water sources to lower contaminant concentration. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 41 When contaminant begins passing through exhausted treatment media. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| --- | --- |
| BACK ¦ Card 44 Reverse Osmosis/Nanofiltration. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 43 No. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 46 Small plastic particles found in the environment and water. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 45 Short-chain PFAS. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 48 No. This remains an emerging contaminant issue rather than a standard federal MCL compliance item. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 47 Yes. Source: Deck: T5 Flash Cards - PFAS and MCLs |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 49 Why are microplastics difficult to regulate? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 50 Name a treatment process that may reduce microplastics. |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 51 1 mg/L equals how many µg/L? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 52 1 µg/L equals how many ng/L? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 53 4 ppt equals what concentration? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 54 10 µg/L equals how many ng/L? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 55 Why are PFAS measurements often difficult to visualize? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 56 PFAS detected above an applicable response level. First step? |

| BACK ¦ Card 50 Coagulation, sedimentation, filtration, or membrane treatment. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 49 Sampling methods, particle sizes, exposure interpretation, and health data are still evolving. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| --- | --- |
| BACK ¦ Card 52 1,000 ng/L. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 51 1,000 µg/L. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 54 10,000 ng/L. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 53 Approximately 4 ng/L in water. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 56 Verify data, notify compliance staff, evaluate treatment/source actions. California-specific PFAS response-level obligations should be checked against Water Operator Source Bibliography. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 55 They occur at extremely small concentrations. Source: Deck: T5 Flash Cards - PFAS and MCLs |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 57 PFAS sample collected by operator wearing waterproof PFAS-containing gear. Concern? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 58 Unexpected PFAS detection at one source only. First investigation? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 59 Why should operators track PFAS regulations regularly? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 60 What is the primary operator responsibility during emerging contaminant events? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 61 PFOA MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 62 PFOS MCL? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 63 Arsenic MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 64 Nitrate MCL? |

| BACK ¦ Card 58 Confirm sample validity and identify source-specific contamination pathways. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 57 Possible sample contamination. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| --- | --- |
| BACK ¦ Card 60 Protect public health while maintaining compliance. Source: Deck: T5 Flash Cards - PFAS and MCLs | BACK ¦ Card 59 Regulatory requirements are rapidly evolving, including EPA 2026 proposed PFAS rule changes and California DDW notification/response-level updates. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 62 4 ppt / 4 ng/L under EPA's 2024 final PFAS NPDWR. Recheck before exam day due to 2026 EPA proposed PFAS rule changes. Source: Water Operator Source Bibliography. | BACK ¦ Card 61 4 ppt / 4 ng/L under EPA's 2024 final PFAS NPDWR. Recheck before exam day due to 2026 EPA proposed PFAS rule changes. Source: Water Operator Source Bibliography. |
| BACK ¦ Card 64 10 mg/L as nitrogen. Source: Water Operator Source Bibliography. | BACK ¦ Card 63 10 µg/L. Source: Water Operator Source Bibliography. |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 65 Nitrite MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 66 TTHM MCL? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 67 HAA5 MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 68 Bromate MCL? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 69 Lead Action Level? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 70 Copper Action Level? |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 71 Gross Alpha MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 72 Combined Radium MCL? |

| BACK ¦ Card 66 80 µg/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 65 1 mg/L as nitrogen. Source: Water Operator Source Bibliography. |
| --- | --- |
| BACK ¦ Card 68 10 µg/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 67 60 µg/L. Source: Water Operator Source Bibliography. |
| BACK ¦ Card 70 1.3 mg/L. Source: Water Operator Source Bibliography and Water Operator Source Bibliography. | BACK ¦ Card 69 Transitional/action-level item. Older/current LCR summary material commonly lists 15 ppb / 15 µg/L; EPA's newer NPDWR/LCRI context lists 0.010 mg/L / 10 µg/L. See Lead and Copper Rule Transition Note and verify California exam expectation before exam day. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 72 5 pCi/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 71 15 pCi/L. Source: Water Operator Source Bibliography. |

| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 73 Uranium MCL? | FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 74 Fluoride MCL? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - PFAS and MCLs ¦ Card 75 Which contaminant category is currently one of the hottest topics in drinking water regulation? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 For drinking-water compliance monitoring, what source takes precedence over EPA method-summary tables? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Are manufacturer inserts and product catalogs the highest authority for compliance monitoring? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why should an operator avoid saying “this kit is approved” without context? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Under RTCR, is routine total coliform/E. coli monitoring usually presence/absence or density/count based? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is the standard RTCR microbiological sample volume? |

| BACK ¦ Card 74 4.0 mg/L. Source: Water Operator Source Bibliography. | BACK ¦ Card 73 30 µg/L. Source: Water Operator Source Bibliography. |
| --- | --- |
| BACK ¦ Card 0 The current CFR and the incorporated-by-reference approved method text. EPA tables are useful summaries, but legal compliance is controlled by the CFR, approved method version, and state primacy agency requirements. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 75 PFAS and related emerging contaminants. Source: Deck: T5 Flash Cards - PFAS and MCLs |
| BACK ¦ Card 0 A kit or method may be approved only for certain analytes, matrices, rules, states, method versions, or reporting contexts. Always verify the approved method and use case. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 No. They are useful for method/product understanding, but compliance use must match CFR-approved methods, exact method versions, state requirements, and certified-lab SOPs. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 100 mL. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Presence/absence is sufficient for routine RTCR total coliform/E. coli compliance. Density is not required for routine RTCR samples. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |

| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is the maximum collection-to-incubation window commonly emphasized for RTCR samples? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why is sodium thiosulfate used in bottles for chlorinated microbiological samples? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is the difference between CFU and MPN? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is HPC used for at an operator-study level? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is precision? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is accuracy? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Can data be precise but inaccurate? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is a blank used for? |

| BACK ¦ Card 0 It neutralizes chlorine residual so the disinfectant does not continue killing organisms before analysis. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 30 hours maximum from sample collection to incubation. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| --- | --- |
| BACK ¦ Card 0 HPC is a general indicator of heterotrophic bacterial growth and can help track biological activity or water-quality changes. It is not the same as total coliform/E. coli compliance testing. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 CFU means colony-forming units, usually from plate-count methods. MPN means most probable number, a statistical estimate from positive tubes/wells/trays. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 Accuracy is closeness to the true or accepted value, often checked by recovery of a known addition or reference standard. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Precision is reproducibility: how close repeated results are to each other under controlled conditions. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 A blank checks contamination or background contribution from reagents, water, containers, or the method process. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Yes. Repeated results can agree with each other but all be biased away from the true value due to systematic error. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |

| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is a duplicate or replicate used for? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is a spike or known addition used for? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why should calibration and checks be documented? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is an analytical standard? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why should expired reagents generally not be used for compliance or critical testing? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why is reagent water quality important? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why should volumetric glassware be used correctly instead of estimating with beaker marks? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why is the meniscus important in volume measurement? |

| BACK ¦ Card 0 It helps evaluate accuracy/recovery and whether the sample matrix affects the method. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 It checks repeatability/precision of sampling or analysis. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| --- | --- |
| BACK ¦ Card 0 A known-quality material used to calibrate or check a method so results reflect the sample composition reliably. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Documentation proves the instrument or method was under control when results were produced. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 Contaminants in reagent water can add background, interfere with reactions, or bias low-level analyses. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Reagent strength or purity may change over time, causing biased or invalid results. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 Reading the meniscus correctly helps avoid systematic volume error. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Beaker marks are approximate. Volumetric flasks, pipets, burets, and graduated cylinders have defined accuracy/tolerance. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |

| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is a buret commonly used for? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why should an analytical balance be level and protected from drafts/vibration? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is a titration? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is an endpoint in titration? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What can cause titration error? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What does a colorimetric test measure? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What is a turbidimetric method based on? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 What do pH and ion-selective electrodes require before reliable use? |

| BACK ¦ Card 0 Drafts, vibration, and leveling errors can affect mass readings. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Delivering measured titrant volume during titration. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| --- | --- |
| BACK ¦ Card 0 The method-defined point where the reaction is considered complete, often indicated by color change or instrument response. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 A method where a reagent is added until an endpoint is reached; the volume used is converted to a concentration/result. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 It measures color intensity from a chemical reaction, visually or by instrument, to estimate concentration. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Wrong sample volume, wrong titrant strength, misread endpoint, air bubbles, expired reagent, poor mixing, or wrong units. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 Proper calibration, clean/properly stored probes, stable readings, and appropriate standards. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 Measuring cloudiness/light scatter caused by suspended or precipitated particles. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |

| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Which common lab method family is used for hardness in many training references? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Which method family is commonly associated with alkalinity? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Which method family is commonly associated with residual chlorine in operator testing? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Which field/lab parameter is commonly measured by nephelometric methods? |
| FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Do these lab flashcards replace a site SOP or certified lab method? | FRONT ¦ T5 Flash Cards - Lab Procedures and Methods ¦ Card 0 Why should an operator know lab-method concepts even if a certified lab performs the test? |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is a cross-connection? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is backflow? |

| BACK ¦ Card 0 Titration, often potentiometric or color-endpoint depending on method. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 EDTA titration. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| --- | --- |
| BACK ¦ Card 0 Turbidity. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 DPD colorimetric methods and amperometric titration are common method families; compliance use depends on approved method/context. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 Operators must understand sampling, preservation, result meaning, invalidation risks, reporting urgency, and operational response. Source: Deck: T5 Flash Cards - Lab Procedures and Methods | BACK ¦ Card 0 No. They are operator study aids only. Always follow approved methods, site SOPs, SDS, permits, and regulatory requirements. Source: Deck: T5 Flash Cards - Lab Procedures and Methods |
| BACK ¦ Card 0 Unwanted reverse flow into the potable water system. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 A physical or potential connection between potable water and a source of unknown or lesser quality. Source: Deck: T5 Flash Cards - Cross Connection Control |

| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is backsiphonage? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is backpressure? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What controls protection selection? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is containment protection? |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is isolation protection? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What is the strongest physical protection method? |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What does RP stand for? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What does DC stand for? |

| BACK ¦ Card 0 Reverse flow caused by downstream pressure greater than supply pressure. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 Reverse flow caused by reduced supply pressure. Source: Deck: T5 Flash Cards - Cross Connection Control |
| --- | --- |
| BACK ¦ Card 0 Protection at the service connection to protect the public water system. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 Hazard level, backflow condition, local program rules, approved lists, and installation constraints. Source: Deck: T5 Flash Cards - Cross Connection Control |
| BACK ¦ Card 0 A properly installed air gap. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 Protection at or near the hazard inside the facility. Source: Deck: T5 Flash Cards - Cross Connection Control |
| BACK ¦ Card 0 Double check valve assembly. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 Reduced-pressure principle assembly. Source: Deck: T5 Flash Cards - Cross Connection Control |

| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What does PVB protect against? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What does AVB protect against? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Is a dual check the same as a testable DC assembly? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What are the main parts of a DC assembly under AWWA C510? |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What standard covers double check valve backflow prevention assemblies? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Does AWWA C510 mean AWWA certifies or approves every product? |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Why are test cocks required on testable assemblies? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 What does the flow arrow on an assembly show? |

| BACK ¦ Card 0 Backsiphonage only, usually point-of-use and not continuous pressure. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 Backsiphonage only, not backpressure. Source: Deck: T5 Flash Cards - Cross Connection Control |
| --- | --- |
| BACK ¦ Card 0 Two independently operating check valves, two shutoff valves, and four properly placed test cocks. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 No. A dual check is not equivalent to a testable double check valve assembly. Source: Deck: T5 Flash Cards - Cross Connection Control |
| BACK ¦ Card 0 No. AWWA standards describe minimum requirements and do not constitute product endorsement, testing, certification, or approval. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 ANSI/AWWA C510. Source: Deck: T5 Flash Cards - Cross Connection Control |
| BACK ¦ Card 0 The required direction of flow through the assembly. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 They allow field testing of the assembly. Source: Deck: T5 Flash Cards - Cross Connection Control |

| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Why must assemblies be accessible? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Why does RP drainage matter? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Why is an unprotected bypass a problem? | FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Why is freeze/flood protection important? |
| FRONT ¦ T5 Flash Cards - Cross Connection Control ¦ Card 0 Who controls actual testing and reporting requirements? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What is the main purpose of a drinking water distribution system? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Name common components of a distribution system. | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why are looped distribution systems generally preferred over dead-end layouts? |

| BACK ¦ Card 0 RP assemblies may discharge through the relief valve, so drainage and location must be planned. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 They must be testable, repairable, and maintainable. Source: Deck: T5 Flash Cards - Cross Connection Control |
| --- | --- |
| BACK ¦ Card 0 Environmental damage can impair an assembly. Source: Deck: T5 Flash Cards - Cross Connection Control | BACK ¦ Card 0 It can defeat the intended protection. Source: Deck: T5 Flash Cards - Cross Connection Control |
| BACK ¦ Card 0 To deliver potable water from source, treatment, or storage facilities to customers while maintaining adequate pressure, flow, and water quality. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 The local authority having jurisdiction and applicable state/program rules. Source: Deck: T5 Flash Cards - Cross Connection Control |
| BACK ¦ Card 0 Looped systems improve circulation, redundancy, pressure support, and operational flexibility. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Mains, transmission lines, storage, pumps, valves, hydrants, meters, services, controls, and appurtenances. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |

| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What water-quality issue is common in dead ends or low-flow areas? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does AWWA G200 support in the vault? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Do AWWA standards override laws, regulations, or local codes? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What are transmission lines? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why does pipe material selection matter? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does pipe C value relate to? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What is a storage facility used for in distribution? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why are valves important in a distribution system? |

| BACK ¦ Card 0 Distribution-system operation and management program concepts, including water quality, O&M, documentation, employee training, and internal auditing. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Increased water age, disinfectant residual loss, sediment accumulation, and possible taste/odor or microbial concerns. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| --- | --- |
| BACK ¦ Card 0 Larger pipelines used to move water between source, treatment, storage, and distribution areas. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 No. Applicable law, regulation, code, and authority having jurisdiction control. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 Hydraulic roughness/friction characteristics used in flow and head-loss calculations. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Material affects pressure capacity, corrosion, installation, water quality, cost, service life, and compatibility with site conditions. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 They allow isolation, control, flushing, maintenance, pressure management, and emergency response. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Balancing demand, maintaining pressure, providing emergency/fire supply, and supporting operations. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |

| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What is the primary purpose of a fire hydrant? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What type of hydrant is intended for climates where freezing temperatures occur? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What AWWA standard covers dry-barrel fire hydrants? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why should non-fire use of hydrants be restricted and controlled? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 If a hydrant cannot be operated by one person using a proper hydrant wrench, what should happen? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What AWWA manual is commonly paired with C502 for hydrant field maintenance? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why is distribution-system disinfection important? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What are common contamination routes during water main work? |

| BACK ¦ Card 0 Dry-barrel fire hydrant. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Public fire protection service. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| --- | --- |
| BACK ¦ Card 0 To keep hydrants in satisfactory working condition for fire emergencies and protect water quality. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 ANSI/AWWA C502. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 AWWA Manual M17, Fire Hydrants: Installation, Field Testing, and Maintenance. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 It should be repaired rather than forced with excessive leverage. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 Soil, sediment, foreign water, trench water, runoff, dirty appurtenances, exposed pipe interiors, and poor sanitary practices. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Distribution deficiencies can contribute to waterborne disease; disinfection and sanitary practices reduce contamination risk. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |

| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Name the basic sequence for new pipeline disinfection work. | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What are common storage facility contamination routes? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What AWWA standards are commonly tied to water main and storage facility disinfection? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why is bacteriological testing part of pipeline/storage disinfection? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What are common chemicals used for pipeline/storage disinfection? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Which free chlorine species is more effective as a disinfectant? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does higher pH do to free chlorine effectiveness? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does M20 support in this vault? |

| BACK ¦ Card 0 Open/missing vents, hatches, doors, roof openings, inspection work, foreign matter, and sanitary defects. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Inspection, sanitary construction, flushing/cleaning, backflow prevention during installation, chlorination, final flushing, bacteriological testing, connection, and documentation. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| --- | --- |
| BACK ¦ Card 0 It helps verify sanitary conditions before the facility is returned to potable service. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 AWWA C651 for water mains and AWWA C652 for water-storage facilities. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 Hypochlorous acid, HOCl. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Liquid chlorine, sodium hypochlorite, and calcium hypochlorite. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 Chlorination/chloramination principles, chemical properties, feed systems, safety, strategies, and residual testing topics. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Higher pH shifts free chlorine toward hypochlorite ion, which is less effective than hypochlorous acid. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |

| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does AWWA B403 cover? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What is aluminum sulfate commonly used for in treatment? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does AWWA B600 cover? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does AWWA B604 cover? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What is activated carbon commonly used for? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does AWWA B112 cover? |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What does AWWA B114 cover? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Which membranes generally remove dissolved salts better: MF/UF or RO/NF? |

| BACK ¦ Card 0 As a coagulant to help destabilize particles and improve clarification/filtration. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Aluminum sulfate, including liquid, ground, or lump forms. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| --- | --- |
| BACK ¦ Card 0 Granular activated carbon. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Powdered activated carbon. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 Microfiltration and ultrafiltration membrane systems. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Adsorption of taste-and-odor compounds and some organic contaminants. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 0 RO/NF; MF/UF are primarily particle/microbial barrier processes, while RO/NF address dissolved material to a greater degree. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 Reverse osmosis and nanofiltration systems for water treatment. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |

| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why is the Water Sources uploaded file caveated? | FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 Why should product standards be paired with operator/training references? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Distribution Source Hydrants and Disinfection ¦ Card 0 What controls final installation, operation, and compliance decisions? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 1 What is the main federal drinking-water law for public water systems? |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 2 Which federal regulation part contains National Primary Drinking Water Regulations? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 3 Which federal regulation part is associated with state primacy and implementation? |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 4 Which federal regulation part contains National Secondary Drinking Water Regulations? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 5 Where are California drinking-water MCLs and monitoring requirements mainly found? |

| BACK ¦ Card 0 Product standards define minimum requirements for products/systems; they do not fully explain operations, troubleshooting, or site-specific design. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection | BACK ¦ Card 0 It includes valid AWWA front matter and table of contents, but also suspicious sales/testbank pages and unrelated parsed text, so integrity review is needed. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| --- | --- |
| BACK ¦ Card 1 Safe Drinking Water Act (SDWA). Source: EPA-SDWA. | BACK ¦ Card 0 Current regulations, state/local requirements, authority having jurisdiction, engineering design, manufacturer instructions, and utility SOPs. Source: Deck: T5 Flash Cards - Distribution Source Hydrants and Disinfection |
| BACK ¦ Card 3 40 CFR Part 142. Source: ECFR-40-CFR-142. Review flag: verify section-level details before final release. | BACK ¦ Card 2 40 CFR Part 141. Source: ECFR-40-CFR-141. |
| BACK ¦ Card 5 CCR Title 22. Source: CA-LAWBOOK / CA-TITLE-22. Review flag: section-level Title 22 audit still needed. | BACK ¦ Card 4 40 CFR Part 143. Source: ECFR-40-CFR-143. Review flag: verify section-level details before final release. |

| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 6 What is the current California source for cross-connection control program requirements? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 7 What rule family covers total coliform, E. coli findings, assessments, and corrective action? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 8 What rule family covers TTHM and HAA5? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 9 Which rule family covers filtration, turbidity, CT, Giardia, viruses, and Cryptosporidium? |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 10 What regulatory concept is central to lead and copper compliance? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 11 What principle is used for source-water blending calculations? |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 12 Source A is 2 MGD at 8 mg/L nitrate and Source B is 1 MGD at 2 mg/L nitrate. What is the blended nitrate? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 13 A 0.5 MGD source has 12 ug/L arsenic and a 1.5 MGD source has 4 ug/L arsenic. What is the blended arsenic? |

| BACK ¦ Card 7 Revised Total Coliform Rule / bacteriological monitoring framework. Source: EPA-RTCR / Title 22 context. | BACK ¦ Card 6 Cross-Connection Control Policy Handbook (CCCPH), which replaced the old Title 17 cross-connection regulations. Source: CA-CCCPH. |
| --- | --- |
| BACK ¦ Card 9 Surface Water Treatment Rule family. Source: EPA-SWTR. | BACK ¦ Card 8 Stage 1 and Stage 2 Disinfectants and Disinfection Byproducts Rules. Source: EPA-DBPR. |
| BACK ¦ Card 11 Flow-weighted concentration from each source. Source: Deck: T5 Flash Cards - Regulation Sources and Blending | BACK ¦ Card 10 Action levels and corrosion-control framework, not a simple finished-water MCL model. Source: EPA-LCR. Review flag: LCRI transition context can change. |
| BACK ¦ Card 13 6 ug/L. Calculation: ((0.5 x 12) + (1.5 x 4)) / 2.0 = 6. Source: Deck: T5 Flash Cards - Regulation Sources and Blending | BACK ¦ Card 12 6 mg/L. Calculation: ((2 x 8) + (1 x 2)) / 3 = 6. Source: Deck: T5 Flash Cards - Regulation Sources and Blending |

| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 14 Before changing source-water blend ratios for compliance, what should be checked? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 15 Why can a source blend that lowers nitrate still create an operational concern? |
| --- | --- |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 16 If a source blend increases chlorine demand, what should the operator evaluate? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 17 What is the operator concern if filter turbidity rises after a source blend change? |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 18 A confirmed E. coli positive routine sample is reported. What is the best operator response? | FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 19 What does a Tier 1 public notice generally indicate? |
| FRONT ¦ T5 Flash Cards - Regulation Sources and Blending ¦ Card 20 A compliance sample was analyzed by a lab not accredited for the method. What is the concern? | FRONT ¦ ¦ Card 0 |

| BACK ¦ Card 15 It may increase TDS, hardness, DBP formation potential, corrosion, taste/odor, or treatment demand. Source: Deck: T5 Flash Cards - Regulation Sources and Blending | BACK ¦ Card 14 Permit/operational limits, expected blended quality, required monitoring, documentation, and supervisor/DDW coordination as applicable. Source: Deck: T5 Flash Cards - Regulation Sources and Blending |
| --- | --- |
| BACK ¦ Card 17 Raw water quality and treatment demand may have changed; check coagulant dose, jar testing, filters, and monitoring requirements. Source: Deck: T5 Flash Cards - Regulation Sources and Blending | BACK ¦ Card 16 Disinfectant dose, residual, contact time, affected water, DBP risk, and monitoring/reporting triggers. Source: Deck: T5 Flash Cards - Regulation Sources and Blending |
| BACK ¦ Card 19 A situation with immediate potential health concern requiring rapid public notice. Source: Deck: T5 Flash Cards - Regulation Sources and Blending | BACK ¦ Card 18 Follow the bacteriological rule response plan, notify required contacts, and coordinate repeat/corrective action requirements. Source: Deck: T5 Flash Cards - Regulation Sources and Blending |
| BACK ¦ Card 0 Source: | BACK ¦ Card 20 The result may not be acceptable for compliance reporting. Source: Deck: T5 Flash Cards - Regulation Sources and Blending |